Setting up a two-camera documentary-style interview session requires a well-thought-out plan to ensure a smooth shoot. Here’s a step-by-step guide covering lights, cameras, audio, craft table, and tech station setup.

---

Step 1: Scout the Location & Plan the Layout

Identify a quiet location with good depth for an aesthetic background.

Consider natural light sources and how they’ll change during the interview.

Plan where the interviewer, interviewee, and cameras will be positioned.

---

Step 2: Set Up Lighting

Standard Three-Point Lighting Setup

1. Key Light (Main Light)

Position at 45° from the interviewee’s face.

Use a soft box or diffuser to avoid harsh shadows.

Adjust brightness based on skin tone and desired look.

2. Fill Light (Softens Shadows)

Place opposite the key light, slightly lower in intensity.

Can be a soft LED panel or bounce from a white reflector.

3. Back Light (Hair Light/Separation Light)

Positioned behind and slightly above the interviewee.

It helps separate them from the background.

Optional Enhancements

Practical lights (lamps, candles) for a natural look.

Gels to adjust color balance if mixing daylight & artificial light.

---

Step 3: Set Up Cameras

Camera 1: The Main Shot

Position: Directly in front of the interviewee (slightly off-axis).

Lens Choice: 35mm or 50mm for a natural perspective.

Frame: Medium close-up (chest up).

Focus: On the eyes using face-tracking autofocus or manual focus.

Camera 2: Secondary Angle

Position: At a 45-degree angle from the main camera.

Lens Choice: 85mm or 70-200mm for a tighter shot.

Purpose: Cutaways for dynamic editing.

Tripod & Movement Considerations

Both cameras on tripods for stability.

If movement is needed, consider a slider or gimbal for a cinematic look.

---

Step 4: Set Up Audio

Primary Audio: Boom Mic

Mic Choice: Shotgun mic (e.g., Sennheiser MKH 416, Rode NTG5).

Placement:

Mounted on a boom stand overhead, angled toward the chest.

Keep it out of the frame but close enough for a clean signal.

Backup Audio: Lavaliers

Mic Choice: Wireless lav (e.g., Sennheiser EW 112P, DJI Mic).

Placement: Hidden under clothing with tape or a concealer.

Dual-Recording Tip: One lav records at a lower gain to prevent distortion.

Audio Recorder Setup

Record to an external recorder (Zoom F6, Sound Devices MixPre-3 II).

Check levels (-12dB to -6dB) before rolling.

Syncing Tip: Use a clapperboard or handclap for waveform matching.

---

Step 5: Set Up the Tech Station (Memory Card Dumping & Monitoring)

Location: Near the DP or a quiet corner, away from noise.

Gear Needed:

Laptop with card readers (CFexpress, SD, etc.).

Dual external SSDs (for redundancy).

2 external hard drives with at least 3t TB or more for safe backups.

Workflow:

1. Dump footage after each card fills up.

2. Label & organize footage by camera name & date.

3. Check files before formatting cards.

---

Step 6: Craft Table & Crew Comfort

Location: Near the entrance but away from the shooting area.

Essentials:

Water, coffee, and healthy snacks.

Quick meals (protein bars, fruit, sandwiches).

Trash bags to keep the space tidy.

---

Final Checks Before Rolling

Cameras rolling with fresh cards & batteries

Audio levels set and monitoring engaged

Lighting adjusted for natural skin tones

Crew briefed on interview flow

---

This setup ensures professional, high-quality interviews with efficient workflow management.